VM Request

CHiPS e-Procurement AI Chatbot — Production VM

Date: 18 June 2026

# Configuration

CPU: 20 Cores  — exceeds the 16-core recommendation; extra headroom for BGE-M3/reranker CPU threads and concurrent users

RAM: 64 GB  — recommended minimum is 32 GB (model 6.5 GB + embed/rerank 2.3 GB + OS/app 3 GB + headroom); 64 GB gives comfortable room for growth

OS: Rocky Linux 9.7  (x86_64, kernel 5.14+)

GPU: NVIDIA T4 16 GB vGPU  (strongly recommended)  — without GPU, LLM inference runs on CPU at ~3-8 min/query; T4 brings this to ~90 s/query (current iGPU baseline)

# Storage

100 GB: dedicated OS drive  (mount: /)  — Rocky Linux OS ~5 GB + Python env ~3 GB + Node.js + app code + logs; 100 GB provides adequate headroom

2 TB: separate data drive  (mount: /data)  — Ollama models ~12 GB + BGE-M3/reranker cache ~2.3 GB + Qdrant vector DB + PDF corpus + logs; 2 TB is well above the 500 GB minimum and accommodates long-term corpus and model growth

# Network & Access Requirements

Public IP allocation for application VM

## External access with open ports

80: HTTP  —  Nginx (redirects to 443)

443: HTTPS  —  Nginx TLS termination, proxies to Node.js chat UI

22: SSH admin access  (restrict to known/whitelisted IPs only)

## Internal-only ports  (firewall: block all external access)

3000: Node.js / Express  (JWT auth + chat frontend)

5000: Flask RAG API  (SSE streaming endpoint)

11434: Ollama LLM server

Note: Ports 3000, 5000, and 11434 run without authentication and must communicate over 127.0.0.1 only. Drop all external traffic to these ports at the firewall (firewalld).

# Duration

Permanent

CHiPS e-Procurement Chatbot Development Team   |   18 June 2026